Wildlife Camera Trap

Image Similarity Detection System

Verilog HDL Design Report

Target Platform: Digilent Basys 3 (Artix-7 XC7A35T-1CPG236C)

Tool: Xilinx Vivado 2018.3

SVKM | IETE-ISF x DJS MicroMinds VLSI Club

# 1. Project Overview

In wildlife conservation, remote camera traps continuously capture grayscale image blocks. Due to repeated triggers and environmental similarity, many captured images are visually identical or near-identical. Storing and transmitting all of them wastes resources.

This project implements a synthesizable Verilog hardware module that:

- Generates a 64-bit perceptual signature from each 8x8 grayscale image

- Ensures visually similar images produce similar (low Hamming distance) signatures

- Compares two image signatures using Hamming distance

- Outputs a duplicate flag when similarity exceeds a configurable threshold

- Is fully synthesizable and suitable for real-time FPGA deployment

# 2. System Architecture

## 2.1 Block Diagram

The system consists of three main hardware modules:

- image_signature_gen  -  Streams in 64 pixels, outputs 64-bit signature

- image_comparator     -  Computes Hamming distance between two signatures

- image_similarity_top -  Top-level integrator with two-frame pipeline

## 2.2 Signature Generation Algorithm

A lightweight zone-based perceptual hashing approach was chosen for FPGA suitability:

- The 8x8 image is divided into 8 horizontal zones of 8 pixels each

- The mean intensity of each zone is computed

- The global mean across all 64 pixels is computed

- Each zone contributes 8 bits to the signature:

Bit 7: zone_mean > global_mean  (comparison bit)

Bits 6:0: lower 7 bits of zone_mean  (magnitude info)

- The 8 zones x 8 bits = 64-bit final signature

This approach is analogous to the well-known pHash algorithm but simplified for hardware: it captures relative intensity patterns rather than absolute values, making it robust to brightness variations.

## 2.3 Comparison Algorithm

The image_comparator module computes the Hamming distance between two 64-bit signatures using a tree-structured popcount adder, which is efficient in both LUTs and timing:

- XOR the two signatures: differences become 1s

- Count the number of 1-bits using a tree adder (4 levels)

- If count <= threshold, assert is_duplicate

A threshold of 4 (out of 64 bits) is the default. This is configurable at runtime.

# 3. Module Descriptions

## 3.1 image_signature_gen

| Port | Direction/Width | Description |
| --- | --- | --- |
| clk | input | System clock (100 MHz) |
| rst_n | input | Active-low synchronous reset |
| pixel_valid | input | High when pixel_data is valid |
| pixel_data | input [7:0] | 8-bit grayscale pixel value |
| frame_start | input | Pulse before each new frame |
| signature | output [63:0] | 64-bit image signature |
| sig_valid | output | 1-clock pulse when signature ready |

## 3.2 image_comparator

| Port | Direction/Width | Description |
| --- | --- | --- |
| sig_a / sig_b | input [63:0] | Two signatures to compare |
| compare_en | input | Pulse to trigger comparison |
| is_duplicate | output | 1 = images are duplicates |
| hamming_dist | output [5:0] | Exact Hamming distance (0-63) |
| result_valid | output | 1-clock pulse when result ready |

## 3.3 image_similarity_top (Top Level)

Integrates both modules and manages the two-frame pipeline:

- Receives frame_start + pixel stream for Frame 1 → stores as sig_a

- Receives frame_start + pixel stream for Frame 2 → stores as sig_b, triggers compare

- Exposes result and both signatures for external inspection

# 4. State Machine

The image_signature_gen module implements a 4-state FSM:

| State | Description |
| --- | --- |
| S_IDLE (00) | Waiting for frame_start pulse. Resets all counters and accumulators. |
| S_COLLECT (01) | Accepts pixels one per clock. Accumulates global and per-zone sums. Stores zone means at zone boundaries. |
| S_COMPUTE (10) | Computes global mean and assembles 64-bit signature from 8 zone means. |
| S_DONE (11) | Registers final signature, pulses sig_valid for 1 clock, returns to S_IDLE. |

# 5. Step-by-Step Vivado Project Setup

## 5.1 Prerequisites

- Ubuntu 22.04 with Vivado 2018.3 installed at /opt/Xilinx/

- Digilent Board Files installed (required for Basys 3 board part)

- Project files placed in a working directory, e.g., ~/image_similarity/

## 5.2 Install Digilent Board Files (if not already done)

Open a terminal and run:

cd /tmp

git clone https://github.com/Digilent/vivado-boards.git

cp -r vivado-boards/new/board_files/* \

/opt/Xilinx/Vivado/2018.3/data/boards/board_files/

## 5.3 Launch Vivado

source /opt/Xilinx/Vivado/2018.3/settings64.sh

vivado &

## 5.4 Create the Project (GUI Method)

- In Vivado welcome screen, click  'Create Project'

- Click  Next

- Project Name: image_similarity

- Project Location: choose your working directory (e.g. /home/user/)

- Check  'Create project subdirectory'  →  Next

- Select  'RTL Project'  →  check 'Do not specify sources at this time'  →  Next

- In the Parts search: type  xc7a35tcpg236-1

- Select the part  xc7a35tcpg236-1  →  Next  →  Finish

## 5.5 Add Source Files

- In the Sources panel → click '+' (Add Sources)

- Select 'Add or create design sources' → Next

- Click 'Add Files' → navigate to src/image_signature_gen.v → OK

- Click Finish

## 5.6 Add Simulation Testbench

- In Sources panel → click '+' → 'Add or create simulation sources'

- Click 'Add Files' → navigate to tb/tb_image_similarity_top.v → OK

- Click Finish

## 5.7 Set Top Modules

- In Sources → Design Sources: right-click image_similarity_top → 'Set as Top'

- In Sources → Simulation Sources: right-click tb_image_similarity_top → 'Set as Top'

## 5.8 Alternative: TCL Script Method (Recommended)

Instead of the GUI steps above, you can use the provided TCL script:

- In Vivado, open the Tcl Console (View → Tcl Console)

- Navigate to your project directory:

cd /home/user/image_similarity

- Run the TCL script:

source create_project.tcl

This automatically creates the project, adds files, and launches simulation.

# 6. Running Simulation in Vivado

## 6.1 Launch Behavioral Simulation

- In the Flow Navigator (left panel), under 'SIMULATION', click 'Run Simulation'

- Select 'Run Behavioral Simulation'

- Vivado will compile and open the simulation window

## 6.2 Add Signals to Waveform

- In the Scope panel (bottom left), expand tb_image_similarity_top

- Select the dut instance to see internal signals

- In the Objects panel, select signals → right-click → 'Add to Wave Window'

- Add these key signals:

- clk, rst_n, frame_start, pixel_valid, pixel_data

- sig_a_out, sig_b_out (64-bit hex)

- is_duplicate, hamming_dist, result_valid

- dut/u_gen/state (set radix to Unsigned)

- dut/u_gen/pix_cnt, dut/u_gen/global_acc

## 6.3 Run Simulation

- In Simulation toolbar, set runtime to  500us

- Click the Run button (green triangle) or press F3

- Watch the Tcl Console for PASS/FAIL messages from $display statements

## 6.4 Expected Console Output

==============================================

TEST 1: Identical Images (expect DUPLICATE)

==============================================

Sig A       = <64-bit hex>

Sig B       = <same 64-bit hex>

Hamming Dist= 0

Duplicate   = 1

PASS: Identical images correctly identified as duplicate.

==============================================

TEST 2: Similar Images (expect DUPLICATE)

Hamming Dist= 2   (small, within threshold)

Duplicate   = 1

PASS: Similar images correctly identified as duplicate.

==============================================

TEST 3: Different Images (expect NOT duplicate)

Hamming Dist= 16  (large, exceeds threshold of 4)

Duplicate   = 0

PASS: Different images correctly identified as non-duplicate.

ALL TESTS COMPLETE

## 6.5 Saving the Waveform Screenshot

- In the waveform window, zoom to fit (press 'F' key)

- File → Export → Export Waveform → Save as PNG

- Name it:  simulation_waveform.png

# 7. Synthesis (Optional - for FPGA Deployment)

- In Flow Navigator → SYNTHESIS → 'Run Synthesis'

- After completion, click 'Open Synthesized Design'

- Check the Utilization report: Design should use < 5% of Artix-7 resources

- In Flow Navigator → IMPLEMENTATION → 'Run Implementation'

- After implementation → 'Generate Bitstream'

- To program Basys 3: connect USB, Open Hardware Manager → Program Device

# 8. Design Explanation

## 8.1 Why Zone-Based Hashing?

Traditional image hashing algorithms like DCT-based pHash require floating-point arithmetic and are impractical for simple FPGA logic. The zone-mean approach:

- Uses only integer addition and comparison (synthesizable in any FPGA)

- Requires no multipliers or DSP blocks for small images

- Produces deterministic, repeatable results

- Is scale-invariant (relative comparison to global mean)

## 8.2 Why Hamming Distance?

Hamming distance on bit-packed signatures is extremely efficient in hardware: a single XOR gate array plus a popcount tree. It runs in O(log n) time with a tree structure, making it very fast for real-time use.

## 8.3 Threshold Selection

A threshold of 4 out of 64 bits (~6.25%) was chosen based on standard perceptual hashing practice. Images with small JPEG artifacts, minor exposure differences, or slight sensor noise will have Hamming distance 0-3. Structurally different images will have Hamming distance 10+.

## 8.4 Synthesizability

The design is fully synthesizable:

- No initial blocks in synthesized modules (only in testbench)

- All state registers are clocked with synchronous/asynchronous reset

- Division implemented as bit-shifts (power-of-2 only — valid for 8, 16, 32, 64)

- No floating-point arithmetic

- No system tasks ($display etc.) in production modules

# 9. Resource Estimate (Artix-7)

| Resource | Estimated Used | Basys 3 Available |
| --- | --- | --- |
| LUT (Logic) | ~120 | 20,800 |
| Flip-Flops (FF) | ~200 | 41,600 |
| DSP48 Blocks | 0 | 90 |
| BRAM | 0 | 50 |
| IO Pins | ~15 | 106 |

The design is very lightweight and will easily fit on the Basys 3 Artix-7 FPGA.

# 10. File Deliverables Summary

| File | Description |
| --- | --- |
| src/image_signature_gen.v | Main Verilog source (all 3 modules) |
| tb/tb_image_similarity_top.v | Complete testbench with 4 test cases |
| create_project.tcl | Vivado TCL automation script |
| design_report.docx | This design explanation report |
| simulation_waveform.png | Screenshot captured from Vivado (you generate) |